WEDLOCK & STONE

MARINE SURVEYORS

Capts. Stephen J Wedlock & Jeffrey R Stone;   Ph: (415) 572-3281  e-mail: jrichmond7@juno.com

# SURVEY REPORT #2587

By Request of:	Sailor Sam

The main street.

San Francisco, CA.

Phone:     (415) 123-456

Fax/E-mail:   mickey@mouse.com

### Date:    October 3, 2003

Vessel Name:    “Boris”

Type:    Auxiliary Sail

Service Area:     San Francisco Bay

Official #:    1065359

Certificate aboard / numbers attached:    Yes/Yes

Hull ID #:    XLY - B

Purpose of Survey:    Condition and value for purchase & Insurance.

(Note: this survey is not to be used for transfer to a third party or use other than intended)

Where Surveyed:    In water & drydock @ Bayship and Yacht, Richmond

Rig survey aloft

Attending:    (Buyer)

## Seller:    Private Sale

## Broker/Agent:    2Twin Rivers Marine Ins. Agency

## 7 Marina Plaza

## Antioch, CA. 94509

## Phone:    (925) 777-2171

Fax/E mail:    (925) 779-1749 / twnrvrs@pacbell.net

Estimated Fair Market Value:  $ xx,000             Estimated Replacement Cost:  $ 70,000

(As she lays)                                 (New; similar construction)

1001    BRIDGEWAY    PMB #490               SAUSALITO,   CALIFORNIA    94965

Page 2										   Survey #2587

# VESSEL PARTICULARS

## Builder:    Islander

## Designer:    Bob Finch

Year Built:    1981

Model:    30’ Islander Bahama

LOA:    29’ 11”

Beam:    10’ 00”

Draft:    5’ 00”

Displacement:	    8,230 Lbs.

Ballast (type/amount):    Lead / 3,130  Lbs.

Hull Type:    Fin keel / spade rudder

Material/Color:    Fiberglass / white

### HULL AND SUPERSTRUCTURE:

### Deck:    Non-skid fiberglass

Cabin Sole:    Carpet over FRP; Plywood bilge access panels

### Bulkheads:    Teak veneer plywood

### Hatches&Ports-Gaskets:    Appear good (Note)

Construction Scantlings:    Single skin gel coat over multiple layers of resin impregnated fiberglass.

Superstructure/Layout:    Low trunk forward to aft cockpit / chain locker way forward then vee berths followed by port head w/ stbd vanity. Saloon is next aft w/ port and stbd settees, port side fold up dining table. Galley is aft to port, nav station is aft to stbd w/ quarter berth behind.

General Condition:    Structure appears sound. Vessel is very clean and appears to have been maintained to high standards under the past owner.

#### PROPULSION SYSTEM

General Condition:    Space is clean, equipment appear well maintained - operational

Type:    Diesel

Hours:    707 “By meter”

Manufacturer:   “Volvo Penta”

Model:    MD 7A

Serial Number:    32347

#Cylinders/HP:    2 / 13

Year/Year rebuilt:    Assumed Original

Cooling System:    Raw water

Alarms:    Yes

Gauges:    Full @ helm

Exhaust(s):    Reinforced hose

Silencer(s):    Steel Can type

Reduction Gear:    Yes; Ratio 1.91:1

Belts/Hoses:    Appear good

Shaft:    3/4“ Stainless Steel

Stuffing Box:    Traditional (Note)

Page 3									              Survey #2587

Strut/Bearing:    Single leg Bronze / Cutlass – good

Propeller:    3 bladed Bronze 14LH10 – Spare 2 Blade Bronze 12LH14 aboard

Controls:    Dual Single lever – operate smoothly

Eng. Beds/Mounts:    HD stringers / Appear new - good

#### MECHANICAL SYSTEMS

General Condition:    Good – well maintained – equipment operational and clean

Eng.Rm.Ventilation:    Natural

Insulation:    None

Wheel/Tiller:    SS wheel – “Edson”

Steering Gear:    Cable to quadrant – cables in good condition – sheaves well mounted bearings tight

Emergency Steering:    Tiller

Rudder:    Foam filled FRP – minor separation noted stbd fwd side of rudder (Note)

Rudder Post Gland:    Tube and bearing – bearings have only slight wear. Rudder appears canted to stbd slightly (Note)

Bilge Pump(s)-Manual:    Diaphragm in cockpit

Auto:    “Rule” 2000 Gph

Potable Water-Manual:    Foot pump

Pressure:    “Shur-Flo” w/out accumulator

Air conditioning/Heating:    None

Head(s):    “Groco”  manual

Holding Tank:    Plastic – 20 Gals

Macerator:    None – diaphragm hand pump waste discharge. Head discharges direct to holding tank

USCG Approved:    Yes w/ “Y” valve (33 USC 159)

Showers:    Handheld in head

Sump/Pump:    Drains to bilge

Anchor Windlass:    None

Zincs:    Replaced at haul out

#### ELECTRICAL SYSTEMS

General Condition:    Good – AC systems not tested (Battery charger, water heater)

DC-Batteries/Amps:    (2) 12v Gp27 - New

Boxes:    Plastic w/tops straps

Approved:    Yes (ABYC E 10.7.1 – 10.7.12)

Battery fluid level:    Sealed type

Condition of terminals:    Good

Engine Alternator/Amps:    Est. 55 amps

Charger/Amps:    “Professional Mariner” / 20 amp

Vapor Proof Switch(es):    “Guest”

Panel Location:    At nav station

Circuit Protection:    Magnetic breakers & Fuses

Wiring:    Stranded copper

Installation:    Well loomed

Page 4										   Survey #2587

AC-Shorepower:    30 amp cable

Cabin Lights:    12v - good

Spreader Lights:    12v - good

Navigation Lights:    12v – good – steaming light is out (Note)

Anchor Light:    12v - good

Hot Water Heater:    “Seaward” 6 Gallon - 120vac

Bonding system / Condition:    None

#### GALLEY EQUIPMENT

General Condition:    Good – equipment is well laid out and appears thoughtfully maintained

Sink(s):    Stainless Steel

Refrigeration:    Icebox

Stove Type:    “Gas Systems” CNG

# Burners/Oven:    Two w/ oven

Tank Stowage:    After cockpit locker (Note)

Safety Solenoid:    No - w/ gauge (Note)

#### TANKAGE AND PLUMBING

General Condition:    Spaces and equipment clean - good

# Fuel Tanks/Capacity:    One / 20 Gals

Material:    Aluminum

How Secured:    SS strapping – under cockpit sole

Accessibility/Condition:    Ltd / appears good

Fills/Vents/Overflows:    On deck / topside

Bonding (Fill to Tank):    Yes  (ABYC H 24.15.1)

Fuel Lines/Clamps:    Neoprene - approved

Filters:    “Racor” and secondary

Valves/Manifold:    At tank

# Water Tanks/Capacity:    One / 40 Gals

Material:    Plastic

How Secured:    Framing – stbd settee

Accessibility/Condition:    Ltd / appears good

Fills and Vents:    On deck

Hoses/Clamps:    Good

Filters:    In line at pump

Valves/Manifold:    None observed

Thru-Hull Fittings/Valves:    Bronze and Marelon ball valves – valves appear to have been recently replaced

Approved:    Yes (ABYC H – 27.4, 27.5)

Condition:    Working

Clamps/Hoses:    Double / reinforced

Anti-Siphon Loops:    Yes

Raw water Strainers:    Bronze

Hose Chafing?:    None Observed

Page 5										   Survey #2587

#### ELECTRONICS/NAVIGATION EQUIPMENT

General Condition:    Good – equipment operational except as noted

Compass(es):    “Danforth” 4 inch in pedestal

VHF Radio:    “Horizon”, new “Standard” w/ DSC (not yet installed) and Hand held

Speed Log:    “Datamarine” – power to unit, does not register speed – impeller in backwards (Note)

Wind Indicator:    “Windex”

Depthfinder:    “Datamarine”

GPS/Loran:    None observed

Radar:    “Furuno 1720”

Barometer:    Yes

Ship’s Clock:    6 inch Brass

#### DECK EQUIPMENT

General Condition:    Good – covers appear new, ground tackle adequate to vessel and service

Anchor(s):    CQR 20 Lb. primary; 13Lb Danforth secondary

Chain/Rode(s):    5/16” x 20’ / 1/2” x 200’ Primary – 1/4” x 20’ / 1/2” x 200’ secondary

Bow Roller(s):    On stem

Bow/Stern Pulpit:    1” Stainless Steel / 1” SS

Stanchions/Lifelines:    1” SS / double wire

Docklines:    Misc. braid

Fenders:    Medium pneumatic

Covers:    Dodger, mainsail full covers below – appear new, not inspected

Swim Ladder:    SS

#### SPARS, RIGGING AND SAILS

General Condition:    Spar was inspected aloft. Welds appeared in good condition. Shroud terminals appeared in good condition. All swages appeared good. Gooseneck, boom end, all sheaves and halyards, bails and hardware attachments, were inspected and found in good condition.

Type of Rig/# Masts:    Sloop / One

Material:    Aluminum

Where Stepped:    On deck

Condition of Step:    Good – teak compression post below was in good condition, no cracks or deterioration was noted.

Keel bolts:    Ballast is encapsulated

Standing Rigging:    1x19 SS wire

Condition:    Appears good  aloft and alow

Terminals:    Swaged

Turnbuckles/(pins?):    SS Bronze / yes no

Chainplates/Stem:    SS – bulkheads were found sound and secure w/ no apparent leaks through bedding

Page 6										   Survey #2587

Spinnaker Pole/Reaching Strut:    Aluminum

Running Rigging:    Dacron braid

Condition:    Good

Winches:    “Lewmar” 2x#8, 1x#30ST, 2x #405

Rigging details (vang/traveler,etc.):    “Harken” hardware on deck, “Hood” roller furler, Halyards lead aft, rope vang.

Sail inventory:    Main, roller furling jib, one bag below – Not inspected

#### SAFETY/REQUIRED EQUIPMENT

General Condition:    Good; except as noted

Throwable Device:    Horseshoe &  Lifesling

PFD’s:    USCG Type II (33 USC 175)

VDS:    “Olin” kit  (33 USC 175) expired (Note)

Horn/Bell:    Handheld freon / NA

Radar Reflector:    Yes

Oil/Pollution Placards:    Yes / Yes (33CFR151.9)

Fire Extinguishers:    (3) 3# ABC dry chemical

Condition:    Gauges in green

Fixed Fire System:    “Fireboy” Halon FE241 (Suggested)

CO detector/ Gas sniffer:    “Suggested” / “Suggested”

#### REMARKS AND RECOMMENDATIONS

This Islander 30 was found in above average condition. Bilge and all spaces were found cleans and dry. Equipment where tested was found operational, thoughtfully installed, and well maintained.

The rig, inspected aloft was found in good condition. All running gear, standing rig, and etc. were found in good condition.

The hull and deck were found clean and well maintained with no apparent separation, or delamination. Light non structural osmotic blistering was noted on the bottom. The rudder was noted to have a slight stbd skew.

In as far as may be ascertained from a general inspection,  without making  extensive removals  or opening up to expose ordinarily concealed areas, and without taking borings to determine thickness or soundness of structures and members, or testing for tightness of components. Upon compliance with the following required items, this vessel should represent a sound financial and insurance risk.

Note: The N.F.P.A. National Fire Protection Association, and  ABYC (American Boat and Yacht Council), do not represent legal requirements, only safety standards. Non -conformity can affect insurance.  U.S.C.G Requirements are law by the Federal Boat Safety Act of 1971.

- REQUIRED NOW

- Assure a minimum of (3) visual distress signals are aboard that have future expiration dates. (consider SOLAS standard) (46CFR28.145)

- Assure steaming light is operational.

Page 7										     Survey#2587

AT OWNER’S DISCRETION / IN NEAR FUTURE:

- Suggest regular pressure hose test to all ports and hatches to check watertight integrity and examine rubber gasketing for cracking.

- Shaft and rudder stuffing glands, thru-hull valves and zincs should be serviced as necessary at each haulout.

- Suggest installation of vent in cockpit coaming above CNG.

- Suggest installation of 12v remote activated shut off solenoid.

- Repair/replace speed impeller (impeller is, at present, installed facing aft.

- Replace cracked bilge Pump discharge thru hull flange (at next scheduled haulout). Note that fitting is above LWL.

- Epoxy injection into rudder at separation noted at survey may be undertaken at the next haul out.

SURVEY LIMITATIONS and SCOPE:

1. Parts of most vessels cannot be examined due to inaccessibility, Some removal procedures add greatly to the time involved and, consequently to the cost. Therefore such procedures are not performed unless specifically requested or recommended. Engine surveys and oil analysis are separate surveys and are recommended. Surveyors do not: Test the vessel, hull or tanks for tightness or leaks; 2: Unload cluttered holds or lockers; clean bottoms; operate the vessel.  It is pointed out that where wood decay is involved, it is not unusual for repairs to uncover previously hidden additional decay.

2. We recommend surveyor attendance underway to test gear under working conditions. The vessel was examined in the water and / or  drydock (see page one for details).  The hull, deck, and house were sounded for structural integrity. Sole boards were lifted and accessible drawers and lockers were  opened for inspection. Machinery, electrical and electronic equipment were operated except as noted.

- Attendance underway:    ____JRS_____Declined     ________Accepted

3. The report is confined to the surveyor's opinion as to the general physical condition and estimated value of the vessel. It does not include a determination  as to the seaworthiness of the vessel, nor does it include stability tests necessary to determine such limitations, nor does it attempt to itemize waters unsuitable for the vessel's use.

Signed without prejudice,

Jeffrey R. Stone:      Society of Accredited Marine Surveyors  (SA; Y, SC)

Master, 1600 Gt. Oceans

#### Member ABYC

#### (415) 572-3281

# STATEMENT

CONDITION AND VALUATION SURVEY

30 ’ x $12/ft		      							$360 .00

Rig survey aloft									$100.00

Paid to Jeffrey R. Stone --							$460.00

With thanks and fair winds,

Capt. Jeffrey R. Stone

Jeffrey R. Stone @ 1001 Bridgeway # 490, Sausalito, CA 94965